# Lab 2 Questions

Aim to complete Parts I-II in the first half of the lab session and Parts III-IV in the second half. If you get to Part V, you will deepen your understanding.

## Part I: Introduction

- Create a heading called “Uncertainty.” In this section, record the most important points from your opening discussion about different types of uncertainty.

  - You’ll refer back to this as you take measurements today to reflect on what types of uncertainty are relevant.

- Refer to last week’s written plan for working as a team, and consider how you want to update it (change roles, address any issues).

  - Refer to the Roles Handout as needed.

  - Create a section in your lab notes titled “Team agreement” and write your team agreement for this week.

## Part II: Series and parallel circuits

(If you did some of this last week, this may go faster for you, but don’t skip this section - there are more questions to answer.)

- Create a heading called “Series and parallel circuits” to record responses to the following questions/prompts.

- Build a circuit with two light bulbs in which all of the current that flows through one light bulb also flows through the other light bulb. (Your instructor will share a picture of this kind of circuit.) In this arrangement, the two bulbs are in series.

  - Take a photo and draw a circuit diagram.

  - How does the bulbs' brightness compare? What does that mean about the amount of current through each bulb?

  - What about the amount of current through the battery: how does that compare to the amount of current through each bulb?

  - Trace the flow of current from one side of the battery, through the circuit elements, and back to the other side of the battery. Draw this flow on your circuit diagram.

  - Student 1 says, “Some of the current is used up in the first bulb, and the rest is used up in the second bulb.” Student 2 says, “The bulbs are equally bright so the current through them must be the same.” Do you agree with student 1, student 2, or neither? Why or why not?

  - We may think of a light bulb as presenting an obstacle, or resistance, to the current in the circuit. Thinking of the bulb in this way, how would you expect the total obstacle to the flow, or resistance, to be affected by the addition of bulbs in series?

- Build a circuit with two light bulbs in which the terminals of the light bulbs are connected together, so at each junction, one terminal of the bulb is directly connected to one terminal of the other. (Your instructor will share a picture of this kind of circuit.) In this arrangement, the two bulbs are in parallel.

  - Take a photo and draw a circuit diagram.

  - How does the bulbs' brightness compare to each other?

  - How does the bulbs' brightness compare to the brightness of a single bulb in its own circuit?

  - Trace the flow of current along its path(s) from one side of the battery, through the circuit elements, and back to the other side of the battery. Does the current split and rejoin along the circuit path?

  - Student 1 says, “The current through the battery in each circuit is the same as when there’s only one bulb, because it’s the same battery. That current is divided between the two bulbs in parallel, so those bulbs get half as much current as a single bulb.” Student 2 says, “Each of the bulbs in parallel must be getting the same current as a single bulb, because they’re the same brightness as a single bulb. Therefore the current through the battery must be greater in this case.” Do you agree with student 1, student 2, or neither? Why or why not? What evidence supports your conclusion?

  - We may think of parallel connections as providing additional paths for current to flow through the circuit. Thinking of the bulb in this way, how would you expect the total obstacle to the flow, or resistance, to be affected by the addition of bulbs in parallel?

## Part III: Measuring current

A multimeter is an electrical device that can measure a variety of electrical quantities. The first quantity you will measure is the current. The units of current are Ampères; when using the multimeter to measure current, we may refer to it as an ammeter.

An ammeter measures the current that flows through it. Therefore, when you want to measure current at a particular place in a circuit, you will need to insert the ammeter into the circuit: disconnect a wire or two and reconnect such that the current has to flow through the ammeter. If you add an ammeter to a circuit without disconnecting anything, you have added it incorrectly. Connecting an ammeter incorrectly can damage it, so always feel free to ask an instructor for help if you are unsure about the use of an ammeter.

In a circuit diagram, an ammeter is represented by a circle with an “A” in it.

To use the multimeter as an ammeter, put wires into the “COM” and the “10A” jack (or "20A" jack if that's what yours has). Then turn the knob until it points to the “10A” (or "20A") setting. (Your instructor can show you a photo to guide you.) This setting can measure currents up to 10 A (or 20A), which is much larger than any current you will encounter in this class. (Use DC settings ⎓, not AC settings ∿.)

- Create a heading called “Measuring current” to record responses to the following questions/prompts.

- Measure and record the current going into a single bulb when it is lit.

- Measure and record the current coming out of the same bulb. How does it compare to the current going in?

- Share your measurements with another group. They should be similar; if they’re not, figure out what is going on.

- Is current used up in the light bulb? Explain the evidence that supports your answer.

- What types of uncertainty are relevant to the measurements you have taken? Explain your answer with reference to the types of uncertainty you discussed at the beginning of class.

## Part IV: Voltage

- Create a heading called “Voltage” to record responses to the following questions/prompts.

- Have your instructor show you three multiple-battery circuits (circuits I-III). (Do not build the circuits!) Observe the relative brightness of the bulbs.

  - Rank the bulbs according to the current through each bulb, from largest to smallest. Explain how you can tell by looking at the circuits.

  - Rank the batteries according to the current through each battery, from largest to smallest. Explain how you can tell by looking at the circuits.

  - If a battery is added to a circuit in series, does the current through the circuit increase, decrease, or remain the same? Explain.

- We may think of a battery as the agent that “pushes” or “drives” current through a circuit. The amount of this “push” that a battery provides is called the voltage or the potential difference. You can measure this push with a multimeter on the voltmeter setting.

To use the multimeter as a voltmeter, put leads into the “COM” and the “V” jack (usually on the right). Then turn the knob until it points to the “200V” setting with the straight/dashed line on top. (Your instructor can show you a photo to guide you.)

A voltmeter measures the voltage (amount of “push”) between the two points that its probes touch. To measure the voltage between two points in a circuit, leave the circuit fully connected, and touch one probe to each point. Always feel free to ask an instructor for help if you are unsure about the use of a voltmeter.

  - Use a voltmeter to measure the voltage across each bulb in circuits I-III. (Ignore any negative signs.) Rank the bulbs according to the voltage across them. How does the ranking of the bulbs according to voltage compare to their brightness ranking?

  - Use a voltmeter to measure the voltage across the following elements in circuit III: battery D, battery E, and the pair of batteries D and E. How does the sum of the voltages across D and E individually compare to the voltage across the D-E pair?

  - What types of uncertainty are relevant to the measurements you have taken? Explain your answer with reference to the types of uncertainty you discussed at the beginning of class.

  - Predict what the voltmeter would read if it were connected to the network of bulbs 3 and 4. Explain your reasoning, then check your prediction.

  - Use your observations to devise a rule that relates potential difference across a network of circuit elements that are connected in series to the voltage across each of the elements in the network.

## Part V: Multiple-bulb circuits

If you get to this part, create a new heading called “Multiple-bulb circuits” to record your work.

- Design, and then build, three different circuits using three (and only three) identical bulbs. (DIfferent circuits means the electrical connections are different, not only the physical arrangement.) Explain the brightness of the bulbs that you observe using the ideas you developed above of obstacles to current (resistance) and paths for current.

- Using the ideas developed above, predict and explain the relative brightness of the bulbs in the circuits below.