# Lab 3 Questions

## Part I: Voltage

(If you completed this section last week, skip to Part II.)

- If you haven’t already, create a heading called “Voltage” to record responses to the following questions/prompts. If you started this section last week but didn’t finish, you may continue in the section you already started.

- Have your instructor show you three multiple-battery circuits (circuits I-III). Observe the relative brightness of the bulbs.

  - Rank the bulbs according to the current through each bulb, from largest to smallest. Explain how you can tell by looking at the circuits.

  - Rank the batteries according to the current through each battery, from largest to smallest. Explain how you can tell by looking at the circuits.

  - If a battery is added to a circuit in series, does the current through the circuit increase, decrease, or remain the same? Explain.

- We may think of a battery as the agent that “pushes” or “drives” current through a circuit. The amount of this “push” that a battery provides is called the voltage or the potential difference. You can measure this push with a multimeter on the voltmeter setting.

To use the multimeter as a voltmeter, put leads into the “COM” and the “V” jack (usually on the right). Then turn the knob until it points to the “200V” setting with the straight/dashed line on top. (Your instructor can show you a photo to guide you.)

A voltmeter measures the voltage (amount of “push”) between the two points that its probes touch. To measure the voltage between two points in a circuit, leave the circuit fully connected, and touch one probe to each point. Always feel free to ask an instructor for help if you are unsure about the use of a voltmeter.

  - Use a voltmeter to measure the voltage across each bulb in circuits I-III. (Ignore any negative signs.) Rank the bulbs according to the voltage across them. How does the ranking of the bulbs according to voltage compare to their brightness ranking?

  - Use a voltmeter to measure the voltage across the following elements in circuit III: battery D, battery E, and the pair of batteries D and E.

  - What types of uncertainty are relevant to the measurements you have taken? Explain your answer with reference to the types of uncertainty you discussed at the beginning of class.

  - Predict what the voltmeter would read if it were connected to the network of bulbs 3 and 4. Explain your reasoning, then check your prediction.

  - Use your observations to devise a rule that relates voltage across a network of circuit elements that are connected in series to the voltage across each of the elements in the network.

## Part II: Measuring current and voltage together

- Create a heading called “Measuring both current and voltage” to record responses to the following questions and prompts.

- Set up a single-bulb circuit using the variable power supply instead of a battery. Start with the variable power supply dials turned low to protect your equipment.

  - Use a multimeter to measure the voltage across, and the current through, the bulb.

  - How do the multimeter readings compare to the numbers displayed on the power supply?

  - Observe and/or calculate (1) the precision uncertainty and (2) the statistical uncertainty. Which is greater? (This is the one you should use going forward.)

  - What can you do, if anything, to reduce the uncertainty in your measurements?

- Design an experiment to document a quantitative relationship between current and voltage for your light bulb. The outcome of your experiment will be a graph. Address the following questions in your design:

  - What ranges of values will you explore? How many data points will you collect within that range? Make sure you get enough data points over a wide enough range to see trends in the data.

  - How many repeated trials for each value of the independent variable will you need? Justify your choice.

  - What are your dominant sources of uncertainty? How will you characterize, quantify, and minimize those sources?

  - Does the point (0,0) belong on your graph? In other words, if the voltage across the bulb was zero, would the current through it also be zero? Measure as needed, and add the appropriate data to your graph.

- Conduct your investigation and document your results. Include a photo showing how your circuit components are connected (including any multimeters). Record any changes to your methods.

- Create a full-page graph displaying your results.

  - Here is a Desmos template: https://www.desmos.com/calculator/8hfscmqhqr - or you may find/create one you prefer, in Desmos, Excel, or a program of your choice.

  - This is a major effort! Use a checklist or rubric provided by your instructor to make sure you include all the necessary elements.

- Is the graph linear? If so, determine the equation of the line. If not, explain what the non-linearity means in this case.

- If you have time, obtain another circuit element from your instructor and repeat steps 1-6 (creating a new heading for this section). Discuss the similarities and differences between the light bulb data and this new data.